ASSIGNMENT 3

Q1. Connect one PC with another PC.

1. Connect Two PCs

- Drag 2 PCs into workspace

- Click Connections (⚡ icon)

- Select Copper Cross-Over cable

- Connect:

  - PC1 → FastEthernet0

  - PC2 → FastEthernet0

2. Assign IP Address

On each PC:

- Go to Desktop → IP Configuration

Fill:

PC1:

- IP Address: 192.168.1.1

- Subnet Mask: 255.255.255.0

PC2:

- IP Address: 192.168.1.2

- Subnet Mask: 255.255.255.0

# 3. Test Connection

From PC1:

ping 192.168.1.2

From PC2:

ping 192.168.1.1

Q2. Connect a PC with Router Ethernet port.

1. Physical Connection

- Use Copper Straight-Through cable

- Connect:

  - PC → FastEthernet0

  - Router → FastEthernet0/0 (or GigabitEthernet0/0)

# 2. Configure Router

Go to Router CLI:

enable
configure terminal
interface fastethernet 0/0
ip address 192.168.1.1 255.255.255.0
no shutdown
exit

3. Configure PC

Method (GUI):

Desktop → IP Configuration

- IP Address: 192.168.1.2

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.1.1

# 4. Test Connection

From PC Command Prompt:

ping 192.168.1.1

Q3. Create a network of 2 PCs.

1. Add Devices

- Drag 2 PCs into workspace

2. Connect PCs

- Go to Connections (⚡)

- Choose Copper Cross-Over cable

- Connect:

  - PC1 → FastEthernet0

  - PC2 → FastEthernet0

3. Assign IP Addresses

On PC1 (Command Prompt):

ip 192.168.1.1 255.255.255.0

On PC2 (Command Prompt):

ip 192.168.1.2 255.255.255.0

4. Test Connectivity

From PC1:

ping 192.168.1.2

From PC2:

ping 192.168.1.1

Q4. Connect 2 PCs with a Switch.

1. Physical Connection

- Add:

  - 2 PCs

  - 1 Switch (e.g., 2960)

- Use Copper Straight-Through cable

Connect:

- PC1 → Switch (FastEthernet0/1)

- PC2 → Switch (FastEthernet0/2)

# 2. Assign IP Address (on PCs)

## Using Command Prompt (inside PC)

### On PC1:

ip 192.168.1.1 255.255.255.0

### On PC2:

ip 192.168.1.2 255.255.255.0

3. Switch Configuration (Optional)

By default, switch works without config, but if required:

enable
configure terminal
interface range fastethernet 0/1 - 2
no shutdown
exit

4. Test Connection

From PC1:

ping 192.168.1.2

From PC2:

ping 192.168.1.1

Q5. Create BUS topologies

a)

b)

Q6. Create the following Ring Topology.

Q7. Create the following Ring Topology

Q8. Create the following Tree Topology.