ASSIGNMENT 4

Q.1 Create a pool on the DHCP Server router, named “xyz” and follow all the requirements.

1. Client should use the DNS server with the IP address: 1.2.3.4

2. Client should use the network 192.168.10.0/24

3. Client should renew their IP address after every 3 hours

4. Make sure, Router does not respond to any BOOTP

5. Configure the Client so that it will obtain IP from DHCP server.

1. Configure DHCP on Router

Go to Router CLI:

enable
configure terminal

! Exclude router IP (recommended)
ip dhcp excluded-address 192.168.10.1

! Create DHCP pool
ip dhcp pool xyz
network 192.168.10.0 255.255.255.0
default-router 192.168.10.1
dns-server 1.2.3.4
lease 0 3 0   ! 3 hours lease

exit

! Disable BOOTP service
no ip bootp server

2. Configure Router Interface (IMPORTANT)

interface fastethernet 0/0
ip address 192.168.10.1 255.255.255.0
no shutdown
exit

3. Configure Client (PC)

On PC:

- Go to Desktop → IP Configuration

- Select DHCP

4. Verify

On PC:

ipconfig

You should see:

- IP: 192.168.10.x

- Gateway: 192.168.10.1

- DNS: 1.2.3.4

Test:

ping 192.168.10.1