ASSIGNMENT 5

Q.1 Objective: Configure Static Route in Cisco packet tracer

• Static route tells the device exactly where to send traffic, no matter what.

• Static route is often used when your network has only a few routers or there is only one

route from a source to a destination.

1.Networks:

- Left LAN: 192.168.1.0/24

- Right LAN: 192.168.2.0/24

- Between routers: 10.0.0.0/30

2. Configure IP on Routers

🔹 R1

enable
configure terminal

interface fastethernet 0/0
ip address 192.168.1.1 255.255.255.0
no shutdown

interface serial 0/0/0
ip address 10.0.0.1 255.255.255.252
no shutdown
exit

🔹 R2

enable
configure terminal

interface fastethernet 0/0
ip address 192.168.2.1 255.255.255.0
no shutdown

interface serial 0/0/0
ip address 10.0.0.2 255.255.255.252
no shutdown
exit

3. Configure Static Routes (MAIN PART)

On R1:

ip route 192.168.2.0 255.255.255.0 10.0.0.2

On R2:

ip route 192.168.1.0 255.255.255.0 10.0.0.1

4. Configure PCs

PC1:

ip 192.168.1.2 255.255.255.0
gateway 192.168.1.1

PC2:

ip 192.168.2.2 255.255.255.0
gateway 192.168.2.1

5. Test

From PC1:

ping 192.168.2.2

Q.2 Suppose you want to create 3 subnets of a network having network address

198.168.1.0. In another network with network address 220.168.2.0, you want to

create 2 subnets. For this scenario configure the end devices and routers

accordingly. Connect two different networks (both are subnetted) through static

routing and show the message transmission among hosts of the two networks

1. Subnetting

🔹 Network 1: 198.168.1.0 → need 3 subnets

Borrow 2 bits → /26 (255.255.255.192) → gives 4 subnets

Subnets:

- 198.168.1.0/26

- 198.168.1.64/26

- 198.168.1.128/26

🔹 Network 2: 220.168.2.0 → need 2 subnets

Borrow 1 bit → /25 (255.255.255.128)

Subnets:

- 220.168.2.0/25

- 220.168.2.128/25

2. Topology

- R1 → handles 3 subnets (Network 1)

- R2 → handles 2 subnets (Network 2)

- R1 ↔ R2 connected via serial link

3. Router Configuration

🔹 R1 (3 subnets side)

enable
configure terminal

interface fastethernet 0/0
ip address 198.168.1.1 255.255.255.192
no shutdown

interface fastethernet 0/1
ip address 198.168.1.65 255.255.255.192
no shutdown

interface fastethernet 1/0
ip address 198.168.1.129 255.255.255.192
no shutdown

interface serial 0/0/0
ip address 10.0.0.1 255.255.255.252
no shutdown
exit

🔹 R2 (2 subnets side)

enable
configure terminal

interface fastethernet 0/0
ip address 220.168.2.1 255.255.255.128
no shutdown

interface fastethernet 0/1
ip address 220.168.2.129 255.255.255.128
no shutdown

interface serial 0/0/0
ip address 10.0.0.2 255.255.255.252
no shutdown
exit

👉 If DCE side:

clock rate 64000

4. Static Routing (IMPORTANT)

On R1:

ip route 220.168.2.0 255.255.255.128 10.0.0.2
ip route 220.168.2.128 255.255.255.128 10.0.0.2

On R2:

ip route 198.168.1.0 255.255.255.192 10.0.0.1
ip route 198.168.1.64 255.255.255.192 10.0.0.1
ip route 198.168.1.128 255.255.255.192 10.0.0.1

5. Configure End Devices

Example PCs:

Network 1:

- PC1 → 198.168.1.2 /26, GW 198.168.1.1

- PC2 → 198.168.1.66 /26, GW 198.168.1.65

- PC3 → 198.168.1.130 /26, GW 198.168.1.129

Network 2:

- PC4 → 220.168.2.2 /25, GW 220.168.2.1

- PC5 → 220.168.2.130 /25, GW 220.168.2.129

6. Test Communication

From any PC (e.g., PC1):

ping 220.168.2.2

Q.3. Connect two networks having network address 192.168.3.0 and 224.152.1.0

respectively. Network 1 having 4 and network two having 3 hosts. Connect two

different networks through a router. For this scenario configure the end devices

and routers accordingly and show the message transmission among hosts of the

two networks.

1. Network Planning

You have:

- Network 1: 192.168.3.0 (4 hosts needed)

- Network 2: 224.152.1.0 (3 hosts needed)

Minimum subnet for both = /29 (255.255.255.248)

- Gives 6 usable IPs (enough for 4 and 3 hosts)

# 2. Router Configuration

Go to Router CLI:

enable
configure terminal

! Network 1 interface
interface fastethernet 0/0
ip address 192.168.3.1 255.255.255.248
no shutdown

! Network 2 interface
interface fastethernet 0/1
ip address 224.152.1.1 255.255.255.248
no shutdown
exit

# 3. Configure End Devices

## 🔹 Network 1 PCs (4 hosts)

PC1: ip 192.168.3.2 255.255.255.248
     gateway 192.168.3.1

PC2: ip 192.168.3.3 255.255.255.248
     gateway 192.168.3.1

PC3: ip 192.168.3.4 255.255.255.248
     gateway 192.168.3.1

PC4: ip 192.168.3.5 255.255.255.248
     gateway 192.168.3.1

## 🔹 Network 2 PCs (3 hosts)

PC5: ip 224.152.1.2 255.255.255.248
     gateway 224.152.1.1

PC6: ip 224.152.1.3 255.255.255.248
     gateway 224.152.1.1

PC7: ip 224.152.1.4 255.255.255.248
     gateway 224.152.1.1

# 4. Test Communication

From any PC (example PC1):

ping 224.152.1.2