ASSIGNMENT 7

Objective

- To understand the Link State Routing algorithm (OSPF)

- To configure and verify OSPF routing in a network

- To analyze how Shortest Path First (SPF) algorithm works

Problem Statement

Design and implement the given network topology with three routers (R1, R2, R3), two

LANs, and PCs.

In addition to configuration, analyze route selection using OSPF cost and SPF algorithm.

1.IP Addressing Scheme

LAN1:

- Network: 192.168.1.0/24

- R1: 192.168.1.1

- PC1: 192.168.1.2

LAN2:

- Network: 192.168.2.0/24

- R3: 192.168.2.1

- PC2: 192.168.2.2

WAN Links:

- R1–R2 → 10.0.0.0/30

- R2–R3 → 10.0.0.4/30

2. Router Configuration

🔹 R1

enable
configure terminal

interface fastethernet 0/0
ip address 192.168.1.1 255.255.255.0
no shutdown

interface serial 0/0/0
ip address 10.0.0.1 255.255.255.252
no shutdown
exit

🔹 R2

enable
configure terminal

interface serial 0/0/0
ip address 10.0.0.2 255.255.255.252
no shutdown

interface serial 0/0/1
ip address 10.0.0.5 255.255.255.252
no shutdown
exit

🔹 R3

enable
configure terminal

interface fastethernet 0/0
ip address 192.168.2.1 255.255.255.0
no shutdown

interface serial 0/0/1
ip address 10.0.0.6 255.255.255.252
no shutdown
exit

3. Configure OSPF (MAIN PART)

On R1:

router ospf 1
network 192.168.1.0 0.0.0.255 area 0
network 10.0.0.0 0.0.0.3 area 0

On R2:

router ospf 1
network 10.0.0.0 0.0.0.3 area 0
network 10.0.0.4 0.0.0.3 area 0

On R3:

router ospf 1
network 192.168.2.0 0.0.0.255 area 0
network 10.0.0.4 0.0.0.3 area 0

4 Configure PCs

PC1:

ip 192.168.1.2 255.255.255.0
gateway 192.168.1.1

PC2:

ip 192.168.2.2 255.255.255.0
gateway 192.168.2.1

5. Verify OSPF

On routers:

show ip route

6. Test Communication

From PC1: ping 192.168.2.2