ASSIGNMENT 8

Title: Configuration and Analysis of BGP Routing

1. Objective

- To understand the working of Border Gateway Protocol (BGP)

- To configure External BGP (eBGP) between different Autonomous Systems

- To analyze path selection using BGP attributes

1.IP Addressing Scheme

LAN1 (AS 100):

- Network: 192.168.1.0/24

- R1: 192.168.1.1

- PC1: 192.168.1.2

LAN2 (AS 200):

- Network: 192.168.2.0/24

- R2: 192.168.2.1

- PC2: 192.168.2.2

WAN Link:

- R1–R2: 10.0.0.0/30

  - R1: 10.0.0.1

  - R2: 10.0.0.2

2. Router Configuration

🔹 R1 (AS 100)

enable
configure terminal

interface fastethernet 0/0
ip address 192.168.1.1 255.255.255.0
no shutdown

interface serial 0/0/0
ip address 10.0.0.1 255.255.255.252
no shutdown
exit

🔹 R2 (AS 200)

enable
configure terminal

interface fastethernet 0/0
ip address 192.168.2.1 255.255.255.0
no shutdown

interface serial 0/0/0
ip address 10.0.0.2 255.255.255.252
no shutdown
exit

3. Configure eBGP (MAIN PART)

🔹 On R1 (AS 100)

router bgp 100
neighbor 10.0.0.2 remote-as 200

network 192.168.1.0 mask 255.255.255.0

🔹 On R2 (AS 200)

router bgp 200
neighbor 10.0.0.1 remote-as 100

network 192.168.2.0 mask 255.255.255.0

4. Configure PCs

PC1:

ip 192.168.1.2 255.255.255.0
gateway 192.168.1.1

PC2:

ip 192.168.2.2 255.255.255.0
gateway 192.168.2.1

5. Verify BGP

Check neighbors:

show ip bgp summary

👉 State should be:

Established

Check routes:

show ip route

6. Test Communication

From PC1:

ping 192.168.2.2